Bearing selection and verification

Index

1.	Scope	3

2.	Definitions	3

3.	Acceptability of bearings	3

4.	User instructions and calculation notes	4

4.1.	Sheet “1 - Input data and info”: User Instructions	5

4.2.	Sheet “1 - Input data and info”: Calculation Notes	7

4.3.	BDS report format sheets (“2-…”, “3-…”, “4-…”): User Instructions	9

4.4.	Sheet “5 – Bearing Database”: User Instructions	9

5.	References	10

# SCOPE

This procedure serves as a comprehensive guide for the calculation module Mod.1122 “Bearing Life Calculation Master”. Mod.1122 is designed to perform calculations on antifriction bearings and provides standard report formats when a BDS is required as a job document. The module performs the following calculations:

- Bearing life for thrust bearings.

- Bearing life for radial bearings.

- Combined bearing system life.

- Bearing’s “reference speed” verification.

- NDM factor verification.

All calculations adhere to the reference standard API 610.

This procedure applies to Nova Milanese plant and La Spezia plant.

# Definitions

ABS	Absolute Value (mathematical notation)

BDS	Bearing Data Sheet (document for customer)

NDM	Bearing factor calculated as N (rotational speed) x DM (mean bearing diameter)

RPM	Rotations Per Minute

SOP	Standard Operation Procedure

VS	Vertically Suspended (pump type)

# Acceptability of bearings

Mod.1122 is a tool designed to ensure the reliability and longevity of pump bearings under various operating conditions. This module integrates multiple input data to compute the bearing life for both thrust and radial bearings, the combined bearing system life, and other critical verifications. Axial thrust and radial forces are inputs and must be calculated separately from Mod.1122, and their calculation is not covered by this SOP.

According to API 610, the minimum “bearing system life” should be at least 25,000 hours under continuous operation at rated conditions and at least 16,000 hours at maximum loads and speeds, and in other points inside the pump’s allowable operating region.

For a good thermal behaviour and reliability of the bearing, L10h upper limit of 40,000 h should be used. The module verifies if API standard requirements or a minimum of 40,000 h is respected.

The definitions of bearing system life and L10h are presented in Chapter 4.2.

API 610 also specifies that the NDM factor for individual bearings shall not exceed 500,000 rpm·mm for oil-lubricated and 350,000 rpm·mm for grease-lubricated bearings (ref. chapter 10, Table 10 of API 610 12th edition). The NDM factor must remain within these limits to prevent thermal issues.

In addition to these verifications, the module guides the user in checking whether the actual bearing speed is within acceptable limits. For this purpose, the rated speed must be compared to the Reference Speed of the bearing. The Reference Speed is defined by ISO 281 as the speed over which the bearing, in a standard loading configuration, lubrication and cooling, reaches a temperature increase of 50°C higher than ambient temperature. To adjust the reference speed using actual lubrication type loads on the bearing, the user can use the “SKF Product select” tool available at:

https://productselect.skf.com/rollingbearings/index.html#/single-bearing/search

This tool should be used when the bearing’s operating speed exceeds 75% of its nominal reference speed.

The “SKF Product select” tool calculates the value of “adjusted reference speed”. If the rated speed of the bearing is lower than the adjusted reference speed, then the bearing selection can be considered acceptable without any further cooling device. If the rated speed is higher and the selection of a different size of bearing is not feasible, then it may still be possible to use a bearing operating above its reference speed, provided certain conditions are met. According to the guidelines in the SKF Rolling Bearings (ref. paragraph B5) catalogue, this requires careful consideration of lubrication and cooling strategies. It is strongly recommended to consult an SKF specialist for technical support. SKF can perform a detailed analysis of the selected bearing under actual operating conditions—such as speed, temperature, and load—to assess whether enhanced lubrication or improved cooling can ensure safe and reliable performance.

# User instructions and calculation notes

Mod. 1122 is organized into multiple sheets:

- 1 - Input data and info: this sheet is designated for entering all input data, and calculations are performed accordingly.

- 2 - BDS with worn gaps: BDS report format used when the calculation considers gap clearances in the thrust calculation. Gap clearances are generally considered in multistage pumps calculation or when there is a specific requirement in the job technical specifications. It can be used for all types of pumps, except VS.

- 3 - BDS without gaps: BDS report format used when the gap clearances in the thrust calculation are not considered. It can be used for all types of pumps, except VS.

- 4 - BDS VS pumps: BDS report format specific for VS pumps. In this case, the calculation is carried out without considering gap clearances.

- 5 - Bearing Database: this sheet contains a database of bearings data used for calculations. It can be manually updated, if necessary, and includes special configurations, such as “triplex” bearings (see Chapter 4.1 for more details about this configuration).

- 6 - Misc.: this sheet is a content repository with basic information about various types of pumps and their bearings. It is for informational purposes only and is not involved in the calculations.

## Sheet “1 - Input data and info”: User Instructions

- Section “JOB AND ITEM DATA”: insert the required basic data information. Notes:

  - Speed: insert rated speed.

  - Power: insert pump power, not the hydraulic power.

- Section “PUMP CONSTRUCTION DATA”: fill in the required data. Consider the points below:

  - Thrust angular contact ball bearings are generally installed in matched pairs of single-row bearings. The data provided in '5 - Bearing Database' reflect this paired configuration, with both static and dynamic load ratings representing the combined performance of the bearing pair rather than that of an individual bearing. In accordance with the SKF Rolling Bearings catalogue:

    - Basic static load rating: C0 = 2*C0 single bearing

    - Basic dynamic load rating in back-to-back or front to front:

C = 1.62*Csingle bearing

  - Thrust angular contact ball bearings for OH2 pumps can be mounted in a "triplex" configuration for special applications with high suction pressure and axial thrust. A "triplex" configuration involves installing three bearings of the same type, with a pair mounted in tandem and the third in the opposite direction. There are not suggested values in the SKF catalogue nor in other bearing manufacturers documentation. Nevertheless, for "triplex" configurations, SKF suggests considering both dynamic and static load ratings as equivalent to the ratings of the couple of bearings paired in tandem configuration. This is a conservative assumption since it does not consider the third single bearing contribution to load ratings. The technical data for "triplex" configurations, equivalent to Tandem configuration, are included in "5 - Bearing Database".  Basic dynamic load rating in back-to-back or front to front:

C = 2*Csingle bearing

  - If the size of a bearing is not available in the drop-down list, it is possible to manually add new data for both thrust and radial bearings to the “5 - Bearing Database” sheet. For detailed instructions on how to add bearing data, please refer to Chapter 4.4. After entering the new data, click the drop-down list again to locate and select the newly added bearing.

  - The calculation sheet automatically compares the bearing rated speed with the bearing reference speed. Based on this check, one of the following messages will be displayed, each accompanied by specific comments and recommendations to guide the user in evaluating bearing suitability and taking appropriate action:

    - Reference speed check ok: the bearing is suitable for the application as the bearing’s operating speed does not exceeds 75% of its reference speed. No further verification is required.

    - Attention! Calculate bearing with 'SKF Product select' online: the bearing’s operating speed exceeds 75% of its reference speed, a more detailed analysis with the online tool “SKF Product select” is recommended to rule out potential thermal issues.

    - Warning! Reference speed is exceeded: the bearing is not suitable for the application. The following solutions can be evaluated:

      - Re-evaluate bearing selection: check if a different bearing type (e.g., with a different internal design or material) that supports higher speeds is available.

      - Optimize lubrication or enhance cooling as per Chapter 3

- Section “THRUST BEARING”: fill in required data. Notes:

  - If worn gaps have not been considered in the calculation or if they are not required for the BDS, enter 0 for all loads in the column designated for worn gaps.

- Section “RADIAL BEARING”: fill in required data. Notes:

  - If worn gaps have not been considered in the calculation or if they are not required for the BDS, enter 0 for all loads in the column designated for worn gaps.

  - For VS pumps, enter 0 for all loads since antifriction radial bearings are not included in VS pumps design.

- Section “BREARING LIFE CALCULATION (HOURS)”: no input data is required in this section. It outputs the bearing life calculation and assesses its acceptability. Notes:

  - A green check will appear if the bearing life exceeds 40,000 hours under rated conditions and maximum loads. This result is acceptable even for the most stringent requirements of job technical specifications.

  - A yellow “!” will appear if the bearing life exceeds 25,000 hours under rated conditions or 16,000 hours at maximum loads. This result is acceptable in accordance with API 610, but it may not be acceptable if more stringent job technical specifications are required.

  - A red cross will appear if the bearing life will be less than 25,000 hours under rated conditions or 16,000 hours at maximum loads. This result is not acceptable for API 610 and a re-design for bearings shall be considered.

## Sheet “1 - Input data and info”: Calculation Notes

According to ISO 281, L10h is the number of operating hours at a given speed and load at which 90% of a group of identical bearings are expected to still be operational without fatigue-related failure.

For L10h calculation, it is required the calculation of the “equivalent dynamic load” P.

From SKF Rolling Bearings catalogue, the equivalent dynamic load P acting on angular contact thrust ball bearings is:

- P = Fr + 0.55 * Fa	when	ABS(Fa/Fr) ≤ 1.14

- P = 0.57 * Fr + 0.93 * Fa 	when	ABS(Fa/Fr) > 1.14

For tapered roller bearings is:

- P = Fr + Y1 * Fa 	when	ABS(Fa/Fr) ≤ e

- P = 0.67 * Fr + Y2 * Fa 	when	ABS(Fa/Fr) > e

For spherical roller thrust bearings is:

- P = 0.88 * (Fa + X * Fr )	when	Fr < 0.55 * Fa and the bearing arrangement's run-out does not influence load distribution. In the case of API 610 pump designs, the bearing arrangement is constructed with controlled clearances and precision alignment. Therefore, it is assumed that run-out does not significantly affect the load distribution, and the above formula can be reliably applied.

The definitions of the variables used for the calculation of P are:

- Fa = total axial thrust.

- Fr = radial load per bearing.

- e = limiting value of Fa/Fr for tapered roller bearings (value from SKF Rolling Bearings catalogue).

- Y1 = calculation factor for the axial load (value from SKF Rolling Bearings catalogue for the considered tapered roller bearing).

- Y2 = calculation factor for the axial load (value from SKF Rolling Bearings catalogue for the considered tapered roller bearing).

- X = calculation factor (value from SKF Rolling Bearings catalogue for the considered spherical roller thrust bearing).

Considering the above, the calculation formulas for P can all be put in the following form:

- P = a * Fr + b * Fa when Fa / Fr ≤ limiting value

- P = c * Fr + d * Fa when Fa / Fr > limiting value

Hence, for each bearing, the calculation coefficients a, b, c, d and the limiting values can be tabulated. This tabulation is in sheet "5 - Bearing Database" of Mod. 1122 as a basis for the calculation output in sheet “1 - Input data and info”, section “BEARING LIFE CALCULATION (HOURS)”.

The resulting bearing life in hours (L10h) is:

- L10h = 1000000 / (60 * RPM) * (C / P)3	for ball bearings.

- L10h = 1000000 / (60 * RPM) * (C / P)10/3 for roller bearings.

- for bearing system.

Where:

- RPM = pump rated speed.

- C = dynamic load rating for the considered bearing.

- P = equivalent dynamic load, calculated as the above notes.

## BDS report format sheets (“2-…”, “3-…”, “4-…”): User Instructions

The sheets “2 - BDS with worn gaps”, “3 - BDS without gaps”, and “4 - BDS VS pumps” provide different report formats for the BDS document issue. The calculation results and information from sheet “1 - Input data and info” are organized in a schematic format in each sheet. Before printing the sheet in PDF format, fill in the author, the check, and the approval referents along with the issue date.

All formats include the following information, organized into various blocks:

- Pump data: model/type, RPM, Power, Energy Density calculation, Reference Standards. A simple calculation, implemented in this section, checks whether the energy density is compatible with the use of antifriction bearings in accordance with API 610.

- Bearing data: Model/type for both thrust and radial bearings, maximum allowable speed (referred to as "reference speed" from the SKF Rolling Bearings catalogue), mean bearing diameter, and NDM factor. A simple check, implemented in this section, verifies whether the actual NDM factor complies with the limits specified in API 610 (see Chapter 3 for more details).

- Bearing loads: axial thrust and radial loads are specified for both thrust and radial bearings at rated conditions and maximum loads. Where applicable, these loads are specified for both nominal and worn gaps cases.

- Bearing equivalent loads: bearing equivalent loads are specified for both thrust and radial bearings at rated conditions and maximum loads. Where applicable, these loads are detailed for both nominal and worn gaps cases. The equivalent loads are calculated according to Chapter 4.2.

- L10h bearing life calculation: this section reports the values calculated for L10h in sheet “1 - Input data and info”. An automatic note confirms whether the bearing life calculation is acceptable. The note can be edited according to the specific job requirements if they differ from those of API 610 (i.e., if 40,000 hours are required instead 25,000 hours, etc.).

## Sheet “5 – Bearing Database”: User Instructions

This sheet contains all the bearing data needed to perform the required calculations. Bearing data are divided into two main tables, one for thrust bearings and one for radial bearings.

Each table is divided into few sections depending on the type of bearings (i.e., Angular Contact Ball Bearings in “Thrust Bearings” table).

For each type of bearings, some rows are dedicated to custom bearings data insertion (i.e., “ACBB custom 1”). To add a new bearing not included among the available ones:

- Modify the custom bearing name (i.e., replacing “ACBB custom 1” with “7219 BECB”).

- Only fill in the empty columns. Do not modify any already existing data. Any missing information should be sourced from the SKF Rolling Bearings catalogue. Notes:

  - Pay attention to the units of measurement specified in the tables of “5 – Bearing Database” sheet. Data from SKF Rolling Bearings catalogue shall be converted accordingly.

  - Manually calculate and enter the values in the 'Mean diameter [mm]' column. The mean diameter is defined as the average of the external and internal diameters for the considered bearing.

  - For Tapered Roller Bearings only:

    - Consider bearings of type “Matched bearings arranged face-to-face” from SKF Rolling Bearings catalogue.

    - Column “b” shall be filled with the calculation factor Y1 value from the SKF Rolling Bearings catalogue’s product tables.

    - Column “d” shall be filled with the calculation factor Y2 value from the SKF Rolling Bearings catalogue’s product tables.

  - For Spherical Roller Thrust Bearings only, the calculation coefficient in the column “c” shall be calculated as 0.88*X, where X is the calculation factor specified in SKF Rolling Bearings:

    - X = 1.1 for 292 series.

    - X = 1.2 for 293 series.

    - X = 1.3 for 294 series.

# References

- Mod.1122

- API 610 12th edition

- SKF Rolling Bearings catalogue

- ISO 281